February 16 - 20, 2004	Taipei, Taiwan

|  | INTERNATIONAL AMATEUR RADIO UNION REGION 3 Twelfth Regional Conference |  |
| --- | --- | --- |

Document No. 04/XII/010

Agenda Item: 10.2.3, 12.2

Report From IARU Region 3 Beacon Project Coordinator
Jamie Pye ZL2NN

IARU Region 3 NCDXF/IARU International Beacons

Status of the NCDXF/IARU International Beacons in Region 25 October 2003:

| Call | Location | City/Town | Operator | Status | Logged1 |
| --- | --- | --- | --- | --- | --- |
| ZL6B | New Zealand | Masterton | NZART | OK | 262 |
| VK6RBP | Australia | Perth | WIA | OK | 56 |
| JA2IGY | Japan | Nagoya | JARL | OK | 81 |
| VR2B | Hong Kong | Hong Kong | CRSA, HARTS | OK | 105 |
| 4S7B | Sri Lanka | Colombo | RSSL | OK | 40 |

The appendix on last page has a complete listing of the International Beacons

1. Number of times logged on 28.200MHz between 1 Jan 2003 and 25 Oct 2003 as noted in the hfbeacons mailing list postings

### Other Beacon Matters

UK 5 MHz Beacons: In my report to the Directors September 2002 I reported that both UK amateurs and US had experimental access to a portion of the 5 to 6 MHz band. UK amateurs have retained their experimental access. The UK experimental beacons are now under construction and are to operate on 5290.0kHz.

Interference to the IBP chain: This is particularly bad at times on 14.100 MHz from amateurs using digimodes. The 28.200 MHz IBP frequency does suffer interference from fellow amateurs particularly during contests.

### Beacon Logging

Proposal for an Automatic Beacon Monitor: To create a group of automatic beacon monitoring stations to collect reception data from the 18 NCDXF/IARU International Beacon Project beacons and post the data in graphical form to the web. The equipment would consist of: a multiband omni-directional antenna such as a vertical, a computer controllable receiver and a Pentium 3 class computer with a permanent Internet connection.

The software will control the receiver to monitor the 18 beacons on 5 bands once every 15 minutes. The waterfall plots from each block will be pasted together to create 24-hour reception strips for each station for each band. The strips will be uploaded to a web server for display by station, by band, etc. Summary graphics could display thumbnail information in smaller graphic files. (extracted from the NCDXF web site)

E-mail mailing list: A very valuable source of beacon information is the hfbeacons mailing list. This is a discussion group dedicated to beacon operation and monitoring (mainly on 10m) set up by Steve Reed G0AEV and Martin Harrison G3USF. About 30% of the e-mails on the hfbeacons mailing list refer to the international beacons. (to subscribe: hfbeacons-subscribe@explore.plus.com )

My last report: As this is my last report I wish the 2004 Region 3 Conference successful discussions and conclusions to agenda items. I have attended the Region 3 Conferences as an observer for NZART since Manila 1981 and as the Region 3 Beacon Coordinator since Singapore 1994. I wish my successor as the Beacon Coordination all the best and success in his/her endeavors.

### Appendix

##### IBP Beacon Locations

## NCDXF/IARU Beacon Transmission Schedule at 25 October 2003

| Call | Location | 14.100 | 18.110 | 21.150 | 24.930 | 28.200 | Operator | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4U1UN | United Nations | 00:00 | 00:10 | 00:20 | 00:30 | 00:40 | UNRC | OK |
| VE8AT | Canada | 00:10 | 00:20 | 00:30 | 00:40 | 00:50 | RAC/NARC | OFF1 |
| W6WX | United States | 00:20 | 00:30 | 00:40 | 00:50 | 01:00 | NCDXF | OK |
| KH6WO | Hawaii | 00:30 | OFF | 00:50 | OFF | 01:10 | NOARG, HARC | OK |
| ZL6B | New Zealand | 00:40 | 00:50 | 01:00 | 01:10 | 01:20 | NZART | OK |
| VK6RBP | Australia | 00:50 | 01:00 | 01:10 | 01:20 | 01:30 | WIA | OK |
| JA2IGY | Japan | 01:00 | 01:10 | 01:20 | 01:30 | 01:40 | JARL | OK |
| RR9O | Russia | 01:10 | 01:20 | 01:30 | 01:40 | 01:50 | SRR | ON2 |
| VR2B | Hong Kong | 01:20 | 01:30 | 01:40 | 01:50 | 02:00 | CRSA, HARTS | OK |
| 4S7B | Sri Lanka | 01:30 | 01:40 | 01:50 | 02:00 | 02:10 | RSSL | OK |
| ZS6DN | South Africa | 01:40 | 01:50 | 02:00 | 02:10 | 02:20 | ZS6DN | OK |
| 5Z4B | Kenya | 01:50 | 02:00 | 02:10 | 02:20 | 02:30 | ARSK | OK |
| 4X6TU | Israel | 02:00 | 02:10 | 02:20 | 02:30 | 02:40 | IARC | OK |
| OH2B | Finland | 02:10 | 02:20 | 02:30 | 02:40 | 02:50 | SRAL | OFF3 |
| CS3B | Madeira | 02:20 | 02:30 | 02:40 | 02:50 | 00:00 | ARRM | OK |
| LU4AA | Argentina | 02:30 | 02:40 | 02:50 | 00:00 | 00:10 | RCA | ON2 |
| OA4B | Peru | 02:40 | 02:50 | 00:00 | 00:10 | 00:20 | RCP | OFF4 |
| YV5B | Venezuela | 02:50 | 00:00 | 00:10 | 00:20 | 00:30 | RCV | OK |

Underlined means web site information available

NOTES:

- Hardware problems — may be intermittent

- The call sign is garbled due to a hardware problem

- Equipment stolen

- Transmitter being repaired

____________________