February 16 - 20, 2004	Taipei, Taiwan

|  | INTERNATIONAL AMATEUR RADIO UNION REGION 3 Twelfth Regional Conference |  |
| --- | --- | --- |

Document No. 04/XII/025

Agenda Item: 12.2

Precision Frequency Beacons and Transmissions

A Paper from the New Zealand Association of Radio Transmitters Incorporated (NZART)
for Presentation at the 12th IARU Region 3 Conference Taipei, Taiwan, February 2004

# Introduction

This paper proposes that amateurs consider a network of low power (QRP) precision frequency transmissions for local or regional use.

They could be used for and to promote:

Calibration of equipment

Propagation studies

Time of signal arrival studies

Frequency reference comparisons

Frequency measuring contests

## Background

The Australian HF Time and Frequency service VNG has ceased transmission.  This has left many Region 3 amateurs without a convenient frequency and time reference, within close range and having minimum Doppler shift.  While those that have the capability may use GPS as a frequency reference, this has a cost and some disadvantages.

It has been proposed that amateurs develop and run a schedule of simple low power precision beacons on 80m and 40m to fill this gap.  There is interest in New Zealand in developing such a system.  More work has to be done to define the parameters and operating systems.

This network of beacons is not seen as being in competition with the 5-band NCDXF HF beacon network.  It is intended primarily for precision frequency transmissions at low power levels in the lower bands.

# Possible Technical Specifications

It is envisaged that reception of the QRP signals would use Fast Fourier Transform (FFT) techniques.  A few watts of transmitter power with an unmodulated carrier would be feasible, with hourly identification (ID) using an ARGO 3 second compatible mode (such as MT-HELL or FSK Morse).

Each country in the Region could use a different frequency.  It would be possible for different geographic locations within a country to operate on the same frequency on a schedule for each day of the week.  This would allow local amateurs to receive signals on a stable ground wave path.

The frequency chosen for transmissions would need to avoid interference from television station harmonics and any other known interference sources.

A micro controller could be used to precisely generate the transmitted carrier.  (A companion paper from NZART gives more details on a series of micro controller based development projects).  An accuracy of 1 part in 108 could readily be achieved using a broadcast TV synchronising pulse reference from the network Rubidium standard.  Alternatively, a GPS source could be used.

## Recommendations

# That

- Region 3 Societies note the proposal for a network of low power (QRP) precision frequency transmissions for local or regional use

- Region 3 Conference recommends that member Societies adopt the proposal and develop precision frequency transmission networks

- Member Societies contribute to and participate in the project, with development ideas and inputs sent to precisionbeacons@nzart.org.nz

____________________