February 16 - 20, 2004	Taipei, Taiwan

|  | INTERNATIONAL AMATEUR RADIO UNION REGION 3 Twelfth Regional Conference |  |
| --- | --- | --- |

Document No. 04/XII/069

Agenda Item: 10.2.6, 11.8

# Report from the EMC Coordinator, HL1AQQ
for the IARU/Region 3 Conference

The following report is based on the IEC/CISPR & TC 77 Joint Meeting held from September 22nd to October 4th, 2003 at Jeju, Korea.

Among many EMC subjects discussed during the conference period, our interested areas would be Amateur related subjects, or at least areas those can be influenced even in the future amateur operations.

1) Recently, sub committee I, SC, was formed under CISPR from the former SCs E and G, and it deals with EMC of information technology equipment (ITE), multimedia equipment and receivers. In this SC, the biggest areas of discussion were on PLC (Power Line Communication) and non-invasive testing.  They recommended to put out a CD (Committee Draft) both on non-invasive testing, and on PLT based on CIS/I/44/CD. An amendment to CISPR 22 was discussed at CD level, which is a clarification of its application to telecommunication system on the method of disturbance measurement at ports used for PLC.

2) A study report regarding devices using UWB (Ultra Wideband) technology was circulated at the SC-S.     The WRC (World Radio communication Conference) is considering that UWB devices could produce unacceptable degradation in frequency band allocated to radio communication services, according to the performance and availability requirements of those services.  It also recognizes that UWB technology holds promise for an array of new applications that may provide benefits for users; therefore it resolved to invite ITU-R to continue its studies relating to devices using UWB technology in order to ensure adequate protection of radio communication services. Since UWB devices are radio transmitters or receivers or both, and hence it is not considered as ISM (Industrial, Scientific, and Medical) applications under No. 1.15 of the RR, and are currently under study in ITU-R.

3) Resolution 63(Rev. WRC-03), protection of radio communication services against interference caused by radiation from ISM equipment, was reported at the SC-S. The WRC is considering that ISM equipment generates and uses locally radio frequency energy, whereby outward radiation cannot always be avoided, and there is an increasing amount of ISM equipment working on various frequencies throughout the spectrum, and in some cases a considerable part of the energy maybe radiated by ISM equipment outside its working frequency. Therefore it resolved that, to ensure that radio communication services are adequately protected, studies are required on the limits to be imposed on the radiation from ISM equipment within the frequency bands designated in the Radio Regulations for this use and outside of those bands. CISPR would define limits in Publication 11 on radiation from ISM equipment inside all the bands designated in the Radio Regulations for the use of such equipment.

4) A NWIP (New Work Item Proposal) was proposed on Large LED display boards, which can be hazardous RF noise source to the radio users, and referred to SC-H, and will be discussed this year. Permissible noise limits and in-situ measurement  method are the keys of discussion.

5) 2004 CISPR Conference will be held in Shanghai from September 6 to 17, and 2005 Conference in Cape town, South Africa from October 10 to17, 2005.

Thank you.

_________________________________________________

- The author is the Head of Korean National Committee for IEC/CISPR, a co-opted member of SC-S, and has been participating in the Conference for the last 15 years. HL1AQQ@hanyang.ac.kr

Rhee, Joong Geun,

Professor, E. E. Dept., Hanyang University, 1271 Sa 1 Dong, Ansan, Korea 426-791

+82-11-251-0883 (CP), +82-31-400-5171(O), +82-31-406-8198(F)

____________________