Nlsep02

# INTERNATIONAL  AMATEUR  RADIO UNION.  REGION 3

MONITORING  SYSTEMS  NEWSLETTER.  SEPTEMBER  2002.

The  Indonesian non-amateur activity  being so much on the 40, 30,20 metre bands

and the  inaction of the Government concerned, is indeed causing  a serious concern. The matter is so often reported that we feel hurt to repeat again and again.

In addition to these non-amateurs, there is a definite increase in the number of frequencies used by the fishing trawlers on the high seas, with their amateur radio equipment, instead of the Marine communication equipment, which they are supposed to have. Sri Lankan and Indian boats are often heard using many of our frequencies. The Australian reports indicate that Chinese boats are like wise causing concern. The presence of  many data stations with multi  channel   high speed data traffic with the latest modes of communications are another cause of our worries.

Long range  cordless telephone users are continuing caring none for the interference they are causing to other services.

News from other regions:

Region 1: A spurious transmission from the transmitters of Radio Rossi,on 14106 kHz  was reported. The fundamental was on 15195 kHz.A second spurious was reported on 16284 kHz. The transmitting time was used by Radio Vlanders, Belgium. The spurious were caused by the presence of a very strong  MW Tx on 1089 kHz near the location of the  15195 kHz Tx. Tests were in progress to rectify the fault.      A F1B transmitting signal on 14192 kHz using 500 Hz shift and 36 baud reversal, was reported, which was also reported in Region 2.

Region 2: Unidentified  printer with occasional traffic  in F1B  on 7054 kHz;

A N0N on 14057.2 with a 100 Hz hum;  an A1A  which often repeated 310 daily at 0100 UTC on 14320 kHz  and the many Radio pirates of all types on 24 and 28 MHz bands in A3E and  J3E  modes  were reported.  The last one is believed to be a spy station and the experts in that field have designated it as “ENIGMA M23”

The reports from JARL are not coming through and thus, the report is delayed, for which I am sorry.

Compiled by:

B.L.Manohar “Arasu”

Regional Monitoring Systems Coordinator

WIRELESS  INSTITUTE  OF  AUSTRALIA INTRUDER  WATCH

REPORT  FOR  AUGUST  2002

This month a warm welcome to Dave Jenkin, VK4CEU, who is our new VK4 IW

co-ordinator.  The observers for this month were: VK4CEU,,VK4BTW,,VK6XW

VK8HA.. Many thanks folks...

There has been more Amateur Radio Activity on the 14 Meg Band, especially

below 14100 and the 'Indon Crims' seems to have moved up above 14100 in

amongst the SSB Amateurs who do not seem to worry about Intruders.

There seems to be an increase in Chinese and Taiwanese SSB operation, especially

on the 14 Meg Band. Some may be Taiwanese fishing boats, and others may be

'People Smugglers'.. The Asian CW Stn on bottom end of 14 is not very active there

anymore, reason unknown...

Due to the JA's not being very 'co-ordinated', we have had no reports from Arasu,

VU2UR for July .

--------------------------------------------------------------------------------------------------------------------

Freq   Date  Time  Emm  Degs  RST   Tms  R e m a r k s - D e t a i l s

--------------------------------------------------------------------------------------------------------------------

03510	2408	1235	CW	---	597	---	SZ1  called by ICA.

07000	dly	eveng	SSB	---	S9	mni	Indon Crims

07005	dly	eveng	AJ3L	---	S6/7	17	Indon Crims

07010	dly	  "	RTTY	---	S5	1	Speed/Shift ?

07011	dly	  "	SSB	---	S9	mni	Indon Crims

07012	dly	  "	AJ3L	---	S9	mni	Indon Crims

07012	---	  "	R7	---	S7	1	Quite Strong Sigs

07024	dly	  "	AJ3L	---	S9	mni	Indon Crims

07070	dly	  "	R7	---	S6	1	Multi Channel

07095	dly	  "	R7	---	S3	2	Sawmill 4k wide

07095	dly	  "	RTTY	---	S9	mni	850 shift

14100	2608	2200	A3	330	S8	---	VOA in Indonesian...Gone at 2211 .

14004	oftn	  "	CW	270	S9/4	mni	Asian CW..Strong stn across VK6 and RX stn 330?

14031	dly	  "	PKT	---	S5	mni	Chouschriber...odd voice mix

14035	2108	0836	PKT	280	S3	---	A NEW  SOUNDING Packet

14050	1108	2315	CW	---	S5	---	1 pip every 2 secs..vvv every 30 secs

14057	---	---	---	---	---	---	This Intruder went QRT on 31-07

14057	2308	1240	N0N	330	S9	---	A NEW ONE  Very NOISY Carrier.Could be Chinese

14119	---	  "	Mult	---	S9	----	Multi Chan  500k Spacing

14122	2608	2210	PKT	340	S8	---	Chinese Packet

14135	--	  "	R7	---	S9	mni	Sawmill 4KHz Wide  with TICKS

14141	---	  "	F1	---	S9	1	RTTY Strong Sigs

14177	dly	  "	F1B	135	S9	1	Bearing from TOOWOOMBA VK4

14177	0308	2258	SSB	---	S9	mni	Indon Crims

14185	dly	  "	F1B	---	S9	1	RTTY

14204	dly	  "	F1B	---	S9	1	RTTY

14250	dly	  "	A3E	---	S3	mni	Pyong Yang B/cast

14325	0508	0022	CW	---	S1	mni	Dayly PIPS.. 2 pips every sec.(used to be on 14300)

The 300 plus degrees are from Humpty Doo.

Cheers and all the best de Henry  vk8ha@octa4.net.au

# M-S Rpts for August 2002  NZART

# Freq.		Date	TimeUTC	Mode		Remarks and ID where known

03560		DY	1000-2100	A3E	B-C. R. PYONGYANG

07008		03	1945-		M7B	16 CHNL. PHASE MOD.

07039		DY	H24		A1A	“K” BEACON – RUSSIAN

07054		DY	1930-2100	F1B	250 HZ 100 BDS. QTH=MOSCOW

07070		DY	1935-		A3E	B-C. R.IRAN   PLUS JAMMING

07085		10	0720-		F7B	4 CHNLS. 150 HZ SHIFT. 62 BDS

07094		02	1950-		F7B	8 x 250HZ CHNLS. 150 BDS.

03	1930-

07100		DY	1900-		A3E	B-C. VOICE OF BROAD MASSES

14044		01	2300-		J3E/U	NON-HAM NET. IN HOKKIEN

14046		DY	HX24		F7B	4 CHNL. DATA BURST

14080		07	0835-		F1B	250HZ SHIFT, 81 BDS, ENCRYPTED

14100		DY	HX24		J3E/U	INDONESIAN NON-HAM NETS

14100		05	0630-0715	F7B	8 x 250HZ CHNLS @ 150 BDS

14106		25	0440-		A3E	B-C. UNID. MUSIC.

14104		10	0830-0940	F1B	250HZ. 81 BDS, ENCRYPTED

12	0740-

14116		DY	HX24		F7B	4 CHNL. DATA BURST

14126		DY	HX24		F7B	4 CHNL. DATA BURST

14154		21	0900-		F7B	4 CHNL. DATA BURST

14173		07-11	H24		F7B	8 x 250HZ CHNLS @150 BDS

14250.1	DY	2000-2359	A3E	B-C. R. PYONGYANG. SIMO: 2850.

14280		DY	0930-		A3E	B-C. 2f 7140. R. PYONGYANG

14290		DY	1800-2000	A3E	B-C  2f 7145KHZ. ROMANIAN??

18090		DY	2300-2359	XXX	“HAVANA GURGLE” 3f. BC JAM.

0001-0830

24950-24990	DY	H24		PXX	“CODAR” SEA STATE RADAR.

28000-29000	DY	H24		A3E	C-B TYPE. AND COMMERCIAL

MAINLY CANTONESE. EVERY 5K

AMATEUR  RADIO  SOCIETY  OF INDIA.     MONITORING SERVICE.

### SUMMARY  OF  REPORTS  FOR    AUGUST  2002.   REPORT  No. 96.

## Freq	Date	UTC Hr  Mode	Country               Identification &  remarks

7000	DY	1300	J3EU	INDONESIA		MANY PIRATES IN 2-WAY TRAFFIC

EVERY 5 KHz ABOVE

7MHZ.BAND  MANY FREQS  USED  BY BUBBLE JAMMERS

7001	16	1615	JAM	?			WOBBLE JAMMERS.

7070	29	0225	A3E	INDIA		AIR-TRIVANDRUM-SPLATTER

7070	DY	1613	A3E	IRAN?		PERSIAN B/C

7086	16	1615	A1A	RUSSIA		L9CC CALLING CP17.ALSO 3560 KHz.

14037	27	0205	J3EU	SRI LANKA	FISHING TRAWLERS TRFC

14041	01	0207	J3EU	SRI LANKA	FISHING TRAWLERS TRFC

14122	24	0545	J3EU	SRI LANKA	FISHING TRAWLERS TRFC.

14270	02	1238	A3E	BBC			2X7135  THAI B/C;LATER BURMESE.

14280	DY	0135	A3E	TURKEY?		PERSIAN B/C.

18075	DY	0200	J3EU	INDIA		PARA MIL TRFC.

18100	DY	1230	J3EU	INDIA		SINDHI, KUTCHI TRFC.

21060	17	0415	J3EU	SE ASIA		2-WAY TRFC OM-YL.

BEACONS HEARD:

28020		11	1030	2AEM

28256.8	20	0203	VK3RMH

28209.2	20	0206	KH6AWQ    VY WEAK.

28238		20	0214	ZL3TEN

28202.3	24	0550	ZS1J/B

28030.4	26	0942	UF	2m  Id.

28140.4	26	0947	3GOS	  4m  Id

28360.6	26	0951	L24

28290		26	0955	VR2TEN

28280.5	26	0957	3SZJ

28262		26	0958	VK2RSY

28188		26	1001	JE7YNQ

28256.7	26	1010	VK3RMH

28264.1	26	1011	VK6RWA

18098.8	28	2325	HP1AVS/B

Compiled by:  B.L.Manohar Arasu

MS Coordinator of ARSI, India.

Notes: Items reported from shared bands are for information only.

This newsletter is sent by E-mail to the Regional Coordinators of Region 1,2,  the Directors and R3 HQ, International Coordinator,and to other  contributing members. The hard copy version is being sent to all the National Societies of Region 3, from the R3 HQ.